Department of Computer Science & Engineering

Microprocessor & Computer Architecture

MPCA–Laboratory/Assignment/Hands–on/Project

UE20CS252

Vrushank G                                  PES1UG20CS516                                        SEC-I

| Sl. No. | Programs |
| --- | --- |
| Week No. 3 | - Write a program in ARM7TDMI–ISA to find GCD of two numbers. - Assume operands to be in the CPU registers. .TEXT MOV R0,#6 MOV R1,#4 GCD:CMP R0,R1 BEQ RES BLT LOOP SUB R0,R0,R1 B GCD LOOP:SUB R1,R1,R0 B GCD RES:MOV R2,R0 SWI 0X011 .END Output: - Assume operands in the memory locations. .TEXT LDR R0,=A LDR R1,=B LDR R4,=C LDR R2,[R0] LDR R3,[R1] L: CMP R2,R3 BEQ RES BLT LOOP SUB R2,R2,R3 B L LOOP: SUB R3,R3,R2 B L RES: STR R2,[R4] SWI 0X011 .DATA A:.WORD 125 B:.WORD 25 C:.WORD Output: - Write a program in ARM7TDMI–ISA to find the sum of N data items in the memory. Store the result in the memory location. - Use Pre–indexing addressing mode. .DATA A:.WORD 10,20,30,40,50,60,70,80,90,100 SUM:.WORD 0 .TEXT LDR R0,=A LDR R1,=SUM MOV R2,#0 MOV R3,#4 MOV R4,#1 SUB R0,R0,#4 LOOP: LDR R5,[R0,R3] ;R5<-MEM[R0+R3] ADD R2,R2,R5 ADD R3,R3,#4 ADD R4,R4,#1 CMP R4,#11 BNE LOOP STR R2,[R1] SWI 0X011 Output: - Use Post–indexing addressing mode. .DATA A:.WORD 10,20,30,40,50,60,70,80,90,100 SUM:.WORD 0 .TEXT LDR R0,=A LDR R1,=SUM MOV R2,#0 MOV R3,#4 MOV R4,#1 LOOP: LDR R5,[R0],#4 ADD R2,R2,R5 ADD R4,R4,#1 CMP R4,#11 BNE LOOP STR R2,[R1] .END Output: - Use Auto–indexing addressing mode. .DATA A:.WORD 10,20,30,40,50,60,70,80,90,100 SUM:.WORD 0 .TEXT LDR R0,=A LDR R1,=SUM MOV R2,#0 MOV R3,#4 MOV R4,#1 SUB R0,R0,#4 LOOP: LDR R5,[R0,R3]! ;R5<-MEM[R0+R3],R0<-R0+R3 ADD R2,R2,R5 ADD R4,R4,#1 CMP R4,#11 BNE LOOP STR R2,[R1] .END Output: - Write a program in ARM7TDMI–ISA to find the sum of N data items at alternate [odd or even positions] locations in the memory. Store the result in the memory location. - Use Pre–indexing addressing mode. ;here we are finding the sum of n numbers at odd locations .DATA A:.WORD 10,20,30,40,50,60 SUM:.WORD 0 .TEXT MOV R2,#0 LDR R1,=A LDR R3,=SUM MOV R6,#0 SUB R1,R1,#8 ;change #8 to #4 to find the sum of numbers at even positions LOOP: LDR R4,[R1,#8] ADD R1,R1,#8 ADD R2,R2,R4 ADD R6,R6,#2 CMP R6,#6 BNE LOOP STR R2,[R3] .END Output: - Use Auto–indexing addressing mode. ;here we are finding the sum of n numbers at odd locations .DATA A:.WORD 10,20,30,40,50,60 SUM:.WORD 0 .TEXT MOV R2,#0 LDR R1,=A LDR R3,=SUM MOV R6,#0 LOOP: LDR R4,[R1],#4 ADD R2,R2,R4 ADD R6,R6,#2 CMP R6,#6 BNE LOOP STR R2,[R3] .END Output: - Use Post–indexing addressing mode. ;here we are finding the sum of n numbers at odd locations .DATA A:.WORD 10,20,30,40,50,60 SUM:.WORD 0 .TEXT MOV R2,#0 LDR R1,=A LDR R3,=SUM MOV R6,#0 SUB R1,R1,#8 LOOP:LDR R4,[R1,#8]! ADD R2,R2,R4 ADD R6,R6,#2 CMP R6,#6 BNE LOOP STR R2,[R3] .END Output: - Write a program in ARM7TDMI–ISA to search for an element in an array. Store 00 if the search is unsuccessful and 01 if the search is successful in the register. - Use Linear Search Technique. .TEXT LDR R0,=A LDR R1,=KEY LDR R3,[R1] MOV R4,#1 L:LDR R2,[R0] CMP R2,R3 BEQ RES ADD R0,R0,#4 ADD R4,R4,#1 CMP R4,#6 BNE L MOV R5,#0 B LOOP RES:MOV R5,#01 LOOP: SWI 0X011 .DATA A:.WORD 10,20,30,40,50 KEY:.WORD 60 Output: |