MPCA WEEK5 LAB

Name:Vrushank G

Section:”I”

SRN:PES1UG20CS516

1. Write a program in ARM7TDMI-ISA to multiply 2 matrices of order3.

i.e., implement c[i][j]=c[i][j] + a[i][j] x b[i][j].

a. Use MLA instruction

b. Use MUL instruction

a.

=>CODE:

.DATA

A: .WORD 1,2,3,4,5,6,7,8,9

B: .WORD 9,8,7,6,5,4,3,2,1

C: .WORD 0,0,0,0,0,0,0,0,0

.TEXT

LDR R0,=A

LDR R1,=B

LDR R2,=C

MOV R12,#0

MOV R3,#0

MOV R4,#0

MOV R5,#0

MOV R8,#3

L1: MUL R6,R3,R8

ADD R6,R6,R4

MUL R7,R4,R8

ADD R7,R7,R5

LDR R10,[R0,R6,LSL #2]

LDR R11,[R1,R7,LSL #2]

MUL R7,R10,R11

ADD R12,R12,R7

ADD R4,R4,#1

CMP R4,#3

BNE L1

MUL R9,R3,R8

ADD R9,R9,R5

STR R12,[R2,R9,LSL #2]

MOV R4,#0

MOV R12,#0

ADD R5,R5,#1

CMP R5,#3

BNE L1

MOV R5,#0

ADD R3,R3,#1

CMP R3,#3

BNE L1

SWI 0X011

=>Output:

b.

=>CODE:

.DATA

A: .WORD 1,2,3,4,5,6,7,8,9

B: .WORD 9,8,7,6,5,4,3,2,1

C: .WORD 0,0,0,0,0,0,0,0,0

.TEXT

LDR R0,=A

LDR R1,=B

LDR R2,=C

MOV R12,#0

MOV R3,#0

MOV R4,#0

MOV R5,#0

MOV R8,#3

L1:

MLA R6,R3,R8,R4

MLA R7,R4,R8,R5

LDR R10,[R0,R6,LSL #2]

LDR R11,[R1,R7,LSL #2]

MLA R12,R10,R11,R12

ADD R4,R4,#1

CMP R4,#3

BNE L1

MLA R9,R3,R8,R5

STR R12,[R2,R9,LSL #2]

MOV R4,#0

MOV R12,#0

ADD R5,R5,#1

CMP R5,#3

BNE L1

MOV R5,#0

ADD R3,R3,#1

CMP R3,#3

BNE L1

SWI 0X11

=>Output:

2. Write a program in ARM7TDMI-ISA to find the NORM of a square matrix of

order n

=>CODE:

.DATA

A: .WORD 1,2,7,6,9,5,4,3,8

SUM: .WORD

.TEXT

LDR R0,=A

LDR R9,=SUM

MOV R1,#0

MOV R2,#0

MOV R3,#3

MOV R4,#0

MOV R8,#0

LOOP:MLA R5,R3,R1,R2

MOV R5,R5,LSL #2

LDR R6,[R0,R5]

ADD R4,R4,R6

ADD R2,R2,#1

CMP R2,#3

BNE LOOP

CMP R4,R8

MOVGT R8,R4

ADD R1,R1,#1

MOV R2,#0

MOV R4,#0

CMP R1,#3

BNE LOOP

STR R8,[R9]

SWI 0X011

=>Output:

3. Write a program in ARM7TDMI-ISA to find the ROWSUM of a matrix.

=>CODE:

.DATA

A: .WORD 1,2,3,4,5,6,7,8,9

SUM: .WORD 0,0,0

.TEXT

LDR R0,=A

LDR R9,=SUM

MOV R1,#0

MOV R2,#0

MOV R3,#3

MOV R4,#0

LOOP:MLA R5,R3,R1,R2

MOV R5,R5,LSL #2

LDR R6,[R0,R5]

ADD R4,R4,R6

ADD R2,R2,#1

CMP R2,#3

BNE LOOP

STR R4,[R9],#4

ADD R1,R1,#1

MOV R2,#0

MOV R4,#0

CMP R1,#3

BNE LOOP

SWI 0X011

=>Output: